Appendix V:  Development of Data Acquisition System (DAS)

In evaluating the data acquisition needs, an evaluation of existing DAS’s was conducted. It was determined through the study that no commercial or government developed system (including DASCAR) was available that would meet all the performance requirements.  PATH therefore designed a system that is composed of two distinct systems - one system records engineering data and the other records video data. The engineering data is recorded with a PC based computer.  The computer used is an Industrial Computer Systems 9300™ series bench top computer using ISA/PCI architecture.  This computer records the output from a variety of sensors.  The sensors selected by PATH to capture the environment around the bus include commercially available mono-pulse millimeter-wave radars and scanning infrared lasers.  Both the radar and scanning laser measure distance and azimuth angle for multiple targets.  The radar units are mounted on the front bumper, one on each end, pointing forward.  Ultrasonic sensors were originally used as corner sensors, however they did not work well for two reasons. Firstly, the ground was being picked up as a target as the sensitivity was adjusted to a high level. Secondly, as ultrasound transceiver surface was not water proof it was decided that they were not appropriate as corner sensors.  It was then decided that Denso LIDAR sensors would be better for this role, so several of these were acquired from Denso. Three lidar units are mounted on the bumper.  The units mounted at each end of the bumper are pointing out 20 degrees and the one mounted near the center is pointing straight ahead.  Other sensors record the driver inputs to the bus, such as steering wheel angle, brake line pressure, throttle position, and turn signal activation.  Other sensors include an accelerometer and a GPS system.  The radars, lidars, and GPS data are recorded using RS232 communication protocol.  The remaining sensors are recorded using an analog to digital board and anti-aliasing filters.

Fig. 1 Sensors installed on a bus

Video data is recorded using a commercially available digital video system.  The first digital video recording system implemented saved the video as a series of still images in an encrypted proprietary format.  This limited the level of compression and allowed only three days of data to be collected before the removable hard disks had to be changed.  This also required that the video data first be converted to a standard still-picture format, and then be converted to a standard moving-picture format (MPEG-1).  This was a very time consuming manual process.  The video recorder was not reliable such that it crashed the flash-ROM system several times. A Loronix™ video system was found that offered several improvements over the previous system. This system records video in a standard still format (AVI) and allows for automated conversion to MPEG-1 format.  Much less time is required to convert the video data now that the process is automated. The system also has greater storage capacity than the previous one, allowing one week of data collection before the removable hard disks need to be changed. This system was retrofitted on the first bus and has proven to be much more reliable and easier to use. The video cameras in the originally developed system were too obtrusive, and easily damaged or moved by passengers. A different style of video camera was selected to replace them.  These cameras have a form factor that allowed them to be installed in the destination window of the bus.  This makes them less obtrusive and prevents them from being tampered with. This system records up to six cameras in AVI format onto a PC hard drive.  Four miniature “board cameras” capture video images around the bus.  The cameras capture the front road scene, the left and right front corner road scene, and the passenger compartment of the bus.  The video streams from the four cameras are combined into one video stream by a quad image combiner to extend the hard drive storage capacity.

Synchronization between engineering and video data is very important for later playback. The first item of information for synchronization is the time stamp recorded in the video frame as a title. This time stamp is generated by a title generator which receives the clock time from the engineering computer. This title allows for manual synchronization. The engineering computer also sends three synchronization signals to the video recorder through the alarm inputs. These signals and their triggering time stamps are recorded separately by both the engineering computer and the video recorder. The signals are triggered every one minute, 15 minutes and 60 minutes respectively. By matching the signal records in the engineering data with the records of alarms in the video recorder, time difference between the two computers can be determined. Once the computer time difference is matched, the video clips can be synchronized with the engineering data streams. The synchronization occurs as part of the process of transferring the data from the removable hard disks to a permanent data base storage system. The permanent data base storage system is composed of a Redundant Array of Inexpensive Disks (RAID). Once the data base has been synchronized and broken into small data clips each set of data clips is saved in one folder for easy access.

Fig. 2 System layout on the bus

The data acquisition system has been installed on three buses in the SamTrans fleet.  A fourth system has been prepared for installation on a yet to be determined bus from another agency in the Bay Area. The first system started collecting data in August 2000. The second system started collecting data in April 2001. After the second system started running, the first system was updated with the new design. The third bus started collecting data in January 2002.

Calibration of DAS

The location and direction of some sensors will influence the system performance. Before running the bus out to collect data, the sensors and the entire system must be calibrated. The calibration process involves the following three tasks: 1) measure the location and direction of the sensors, 2) correct the location and direction of some sensors, and 3) examine the system alignment.

This section describes the calibration process of the first DAS on the first bus and gives the results. The 1st section gives the measurements of location and sensor direction. The 2nd section describes the laser radar calibration procedure and results. The 3rd section describes the calibration approaches for cameras. Calibration of system alignment is given in the 4th section. Calibration of other sensors is given in the 5th section. The DAS design was changed after the first DAS was calibrated. However, the calibration process and the techniques presented in this document were conducted to calibrate all the systems. For convenience, the following abbreviations are used.

| Table 1 DAS calibration abbreviations |  |
| --- | --- |
| Sensor Name | Abbreviation |
| passenger side corner camera | P-CAM |
| front-looking camera | F-CAM |
| driver side corner camera | D-CAM |
| passenger side upper ultra-sensor | UP-SONAR |
| passenger side lower ultra-sensor | LP-SONAR |
| passenger side radar | P-RADAR |
| laser radar | LIDAR |
| front-looking ultra-sensor | F-SONAR |
| driver side radar | D-RADAR |
| driver side upper ultra-sensor | UD-SONAR |
| driver side lower ultra-sensor | LD-SONAR |
| Interior-looking camera | I-CAM |
| rear-looking camera | R-CAM |
| rear radar | R-RADAR |
| global positioning system | GPS |

Sensor position

Coordinate systems

To locate the sensors, two reference frames were built on the bus. One is the Front Coordinate System (FCS) and the other is the Rear Coordinate System (RCS). Locations of front sensors, including P-CAM, F-CAM, D-CAM, UP-SONAR, LP-SONAR, P-RADAR, LIDAR, F-SONAR, D-RADAR, UD-SONAR, LD-SONAR and I-CAM, are measured in the FCS. Locations of rear sensors, including R-CAM, R-RADAR and GPS are measured in the RCS. The reference points of the coordinates and the positions of the sensors are illustrated in the following figures. The positive x-axis is horizontally to the left, the positive y-axis is vertically upward, and the positive z-axis is horizontally to forward. The basic dimensions of the bus are:  length = 12200 mm, width = 2750 mm.

Front Sensors

The reference point of the FCS and the locations of the front sensors are illustrated in Fig. 3.

Fig. 3 FCS and front sensors

The reference point is on the front center of the bus. The height of the reference point from the ground is 585mm. The coordinates of the front sensors are listed in the following table.

Table 2 Front sensor locations

| - Sensors | - x (mm) | - y (mm) | - z (mm) | - Angle (Deg) |
| --- | --- | --- | --- | --- |
| - LIDAR | - -836 | - -195 | - 78 | - 1N.A. |
| - P-RADAR | - -1050 | - -132 | - 70 | - N.A. |
| - UP-SONAR | - -1201 | - -97 | - 64 | - 2-36 |
| - LP-SONAR | - -1201 | - -176 | - 64 | - 2-26 |
| - D-RADAR | - 985 | - -135 | - 67 | - N.A. |
| - UD-SONAR | - 1190 | - -95 | - 64 | - 235 |
| - LD-SONAR | - 1190 | - -175 | - 64 | - 226 |
| - F-SONAR | - 790 | - -161 | - 61 | - N.A. |
| - D-CAM | - 396 | - 991 | - -80 | - 314 |
| - F-CAM | - -69 | - 1653 | - -61 | - 313 |
| - P-CAM | - -109 | - 1563 | - -95 | - 325 |
| - I-CAM | - -409 | - 2186 | - -365 | - N.A. |

- 1: N.A. = Not available;

- 2: These are azimuth angles;

- 3: These are tilting angles.

Rear Sensors

The reference point of the RCS and the locations of the rear sensors are illustrated in Fig. 4.

Fig. 4 RCS and rear sensors

The reference point is on the rear center of the bus. The height of the reference point to the ground is 790mm. The coordinates of the rear sensors are listed in the following table.

Table 3 Rear sensor locations

| - Sensors | - x (mm) | - y (mm) | - z (mm) | - Angle (Deg) |
| --- | --- | --- | --- | --- |
| - R-RADAR | - 950 | - -154 | - -39 | - 1N.A. |
| - GPS | - 590 | - 2220 | - 800 | - N.A. |
| - R-CAM | - 500 | - 1500 | - 140 | - 216 |

- 1: N.A. = Not available;

- 2: Tilting angle.

LIDAR calibration

Optical axis orientation

The LIDAR beam is scanning in 2D by rotating a hexagon mirror. The equivalent detection scope is 16 degrees in horizontal and 4.4 degrees in the vertical direction. The equivalent optical axis is defined to originate from the LIDAR lens extending to the center of the detection scope, i.e. eight degrees to both the left and the right margins and 2.2 degrees to both the top and the bottom margins. There are two adjustable screws on the front face of the LIDAR, which can be rotated to adjust the optical axis in 2D (both horizontal and vertical directions). As the LIDAR has been mounted on the passenger side on the 1st bus, to calibrate the LIDAR, we must first adjust the optical axis to an appropriate direction [1].

The LIDAR optical axis is set horizontally to the point on the bus’s longitudinal center line, 50 meters away from the bus front reference point, and vertically 2.2 degrees up with respect to the horizontal plane. The geometric relationship is illustrated in Fig. 5.

Fig. 5 LIDAR calibration geometry

LIDAR calibration procedure

- LIDAR calibration was done by the following procedure.

1. Measure LIDAR lens vertical position (height to ground) H =_0.425__(m).

2. Measure R=_50m_ from bus front reference point along the longitudinal direction.

3. Set the reflector at R=50m with vertical position = H.

4. Adjust both the lower and the higher screws simultaneously, make reported “lateral position” = __0__. Change lateral position to check the adjustment.

Table 4 LIDAR lateral position test

| Actual lateral position | Expected report number | LIDAR report (5th col) |
| --- | --- | --- |
| 6m Left | -60 *.1m | _____-61___ |
| 3m Left | -30 *.1m | _____-30___ |
| 3m Right | 30 *.1m | _____30___ |
| 6m Right | 60 *.1m | _____61___ |

5. Adjust the lower screw, make reported “Vertical Position” changing from smaller to larger numbers thru __12__.

6. Adjust the lower screw to “ –  direction” __0.3-0.5__ rev, make sure that the LIDAR keeps detecting the reflector.

7. Change distance to check the adjustment:

Table 5 LIDAR range test

| Actual distance | Expected report number |  | LIDAR report (1st-2nd col) |  |
| --- | --- | --- | --- | --- |
| 40m | 31*1.28m | 32*.01m | _31__*1.28m | _98__*.01m |
| 30m | 23*1.28m | 56*.01m | _24__*1.28m | _14__*.01m |
| 20m | 15*1.28m | 80*.01m | _16__*1.28m | _48__*.01m |
| 10m | 7*1.28m | 104*.01m | _8___*1.28m | _46__*.01m |

8. Put the reflector at R=10m, with vertical position changing, check the adjustment:

Table 6 LIDAR vertical position test

| Actual vertical position | Expected report number | LIDAR report (9th col) |
| --- | --- | --- |
| H+0.76m | 2 | __2_____ |
| H+0.57m | 3-4 | __4_____ |
| H+0.38m | 6-7 | __5_____ |
| H+0.19m | 9-10 | __6_____ |
| H+0m | 12 | __8_____ |

Camera calibration

Rough adjustment

Three different options of focal length are available: 3mm, 4mm, and 7.5mm. Lenses with different focal length were fitted on the camera heads. Comparing the field of view and selecting the one list that best matches the area of interest around the bus, the optimal fitted focal length was chosen for each camera, as in the following table.

Table 7 Focal length of cameras

| Camera | Focal length |
| --- | --- |
| D-CAM | 4mm |
| F-CAM | 7.5mm |
| P-CAM | 4mm |
| I-CAM | 4mm |
| R-CAM | 7.5mm |

Image plane rotation and optical axis direction of each camera was roughly adjusted by monitoring the video output. The factors of interest while adjusting are: range coverage, azimuthal direction of interest, and consistency between adjacent cameras. The tilting angle of each camera was measured with a level and an angle measure.

Intrinsic and extrinsic parameters calculation

##### Control points

To calibrate the cameras, 20 control points arranged in 4 lines with 5 points in each line were made on a vertically standing black screen. The adjacent lines are 50 centimeters apart. The distance between adjacent points in each line is also 50 centimeters. The screen was put in front of each camera with the points facing the camera. A picture was taken and stored in the computer. The screen was then moved 25 centimeters (for F-CAM and R-CAM) or 20 centimeters (for D-CAM and P-CAM) closer to the camera. This process was repeated until five pictures were taken for each camera. Every time a picture was taken, the position of the screen in the bus coordinate system was marked on the ground and measured later to calculate the control point coordinates.

The pictures were opened in Microsoft Photo Editor™ to read the image coordinates of the control points. We get the coordinates of the control points in the bus coordinate system and their corresponding image coordinates in the picture. Each control point and its image are called a calibration pair. By substituting the coordinates of the calibration pairs in the camera model described below, two equations for each pair were obtained. We can solve the unknown camera parameters from the equations for all pairs in the sense of Least Square Error (LSE).

Camera model

Let  represent the coordinates of a point in the bus coordinate system (FCS or RCS),  represent the coordinates of the point in the camera coordinate system,  and  represent the undistorted and distorted image coordinates of the point respectively, and represent the coordinate read in Microsoft Photo Editor™, i.e. the pixel location with respect to the top-left corner in the image, viz. the computer image coordinate. The relationship between the bus coordinate system and the camera coordinate system is given by [2]:

(1)

where  is a 33 ortho-normal rotation matrix defining the camera orientation and  is a translation vector defining the camera position. The camera coordinate system is transformed to the undistorted image coordinate (2D) system according to the pin-hole model:

(2)

where  is the focal length. The distortion of image coordinates can be modeled by [4]:

(3)

where , are coefficients of tangential distortion, and is the coefficient of radial distortion. The distorted image coordinates are then obtained:

(4.1)

or

(4.2)

The relationship between the distorted image coordinates and the computer image coordinates is given by:

(5)

where  are the distance between the adjacent imaging sensor elements in rows and columns, respectively,  represents the computer image coordinate of the principal point of the image coordinate system.

The model itself is a nonlinear one. The unknown parameters can be categorized into intrinsic and extrinsic, or linear and non-linear parameters, as follows:

Table 8 Parameter table

|  | Linear | Nonlinear |
| --- | --- | --- |
| Intrinsic | ,, | , |
| Extrinsic | , |  |

Calibration procedure

It is hard to solve all the parameters simultaneously from the complete nonlinear camera model. However, if the nonlinear distortion can be neglected, the model becomes linear. Once the linear parameters are known, the nonlinear parameters can be solved from linear equations (3). These properties of the camera model help us to simplify the calibration procedure into the following steps [3]:

Step 1: Assume no distortion, calculate linear model parameters

Step 2: Calculate distortion using the linear parameters estimated in Step 1

Step 3: Calculate nonlinear parameters using the distortion and linear parameters estimated in Step 2

Step 4: Calculate distortion using the linear and nonlinear parameters estimated in Step 2 and 3

Step 5: Subtract the distortion estimated in Step 4 from the image coordinates, loop to Step 1 or terminate

The procedure is terminated when it is convergent. As noise exists in the calibration pair coordinates, the distortion used in Step 5 was multiplied with a positive fraction to confirm convergence. The positive fraction used in our calculation is 0.999.

Calibration results

Control point images

Control point image coordinates estimated with linear-only and nonlinear-plus models together with the actual image coordinates read in Photo Editor™ are illustrated in the following plots, where the ‘o’ signs represent the actual images read in Photo Editor™, the ‘+’ signs represent the images estimated only with linear model, and the ‘x’ signs represent the images estimated with linear plus nonlinear model.

| F-CAM | D-CAM |
| --- | --- |
| P-CAM | R_CAM |
| Fig. 6 Control point images |  |

Distortion

Distortion is calculated and plotted in the following plots.

| x of F-CAM | x of D-CAM |
| --- | --- |
| x of P-CAM | x of R-CAM |
| Fig. 7 x of four cameras |  |

| y of F-CAM | y of D-CAM |
| --- | --- |
| y of P-CAM | y of R-CAM |

Fig. 8 y of four cameras

###### Intrinsic and extrinsic parameters

The intrinsic and extrinsic parameters for different cameras were calibrated and are listed in the following table.

Table 9 Intrinsic and extrinsic parameters

| Parameters | F-CAM |  |  | D-CAM |  |  | P-CAM |  |  | R-CAM |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RT | R1 | R2 | R3 | R1 | R2 | R3 | R1 | R2 | R3 | R1 | R2 | R3 |
|  | -1.0000 -0.0042 -0.0035 -0.0011 -0.9743 -0.2252 -0.0038 -0.2252 0.9743 |  |  | -0.7018 -0.1878 0.6877 0.0535 -0.9763 -0.2092 0.7103 -0.1080 0.6952 |  |  | -0.7390 -0.3102 -0.5965 0.6001 -0.7109 -0.3714 -0.3063 -0.6312 0.7115 |  |  | 0.9921 -0.0326 0.1207 -0.0037 -0.9678 -0.2518 0.1257 0.2496 -0.9602 |  |  |
| TT (m) | -0.0658 1.6280 0.5324 |  |  | 0.3428 1.0512 0.0723 |  |  | -1.1015 1.0498 0.5364 |  |  | -0.5286 1.4410 0.4298 |  |  |
| (i0,j0) | 359.79, 222.63 |  |  | 348.61, 226.08 |  |  | 366.81, 216.45 |  |  | 361.05, 206.93 |  |  |
| (f/wx ,f/ wy)1 | 860.83, 790.62 |  |  | 461.00, 419.96 |  |  | 438.66, 403.40 |  |  | 841.70, 771.48 |  |  |
| (p1,p2,k1)/f | -0.0146 -0.0056 -0.1434 |  |  | -0.0064 -0.0044 -0.2342 |  |  | -0.0006 -0.0048 -0.2447 |  |  | -0.0108 -0.0055 -0.1486 |  |  |

- The imaging element size parameter can not be determined in the calibration procedure. These parameters can be found in camera manufacturer’s specifications. The cameras mounted on bus No.1 are from ELMO. The camera head model is MH42H. The effective image area is 6.54mmx4.89mm.  Effective image pixels are 768 x 494. By simple calculation, the size parameters are:

###### Derived parameter verification

Some parameters can be derived from the calibrated parameters. Location of the cameras can be derived by [5]:

.

Focus length of the cameras can be calculated by simply multiplying with or multiplying with. Angles of the cameras can be calculated from the rotation matrix R:

Tilting angle = -;

Azimuth angle = -

Image rotation =

These derived parameters are listed in the following table.

Table 10 Derived parameters

|  | F-CAM |  | D-CAM |  | P-CAM |  | R-CAM |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Measured | Calculated | Measured | Calculated | Measured | Calculated | Measured | Calculated |
| Location X (mm) | -69 | -57 | 396 | 388 | -109 | -168 | 500 | 523 |
| Location Y (mm) | 1653 | 1706 | 991 | 1023 | 1563 | 1607 | 1500 | 1501 |
| Location Z (mm) | -61 | -152 | -80 | -180 | -95 | -56 | 140 | 111 |
| Focus fx (mm) | 7.5 | 7.32 | 4.0 | 3.92 | 4.0 | 3.73 | 7.5 | 7.15 |
| Focus fy (mm) | 7.5 | 7.83 | 4.0 | 4.16 | 4.0 | 3.99 | 7.5 | 7.64 |
| Tilting Ang (Deg) | 13 | 13.01 | 14 | 12.1 | 25 | 21.8 | 16 | 14.6 |
| Azimuth Ang (Deg) | N.A. | 0.20 | N.A. | -44.7 | N.A. | 40.0 | N.A. | 7.2 |
| Image Rotation Ang (Deg) | N.A. | -0.06 | N.A. | 3.06 | N.A. | 31.0 | N.A. | -0.21 |

### System alignment

The purpose of system alignment is to determine the inter-relationship of multiple sensors in the system. Three sensors, LIDAR, P-RADAR and D-RADAR, are considered. Thirteen locations were marked on the ground in front of the bus. A microwave reflector and a laser reflector were used as targets for the sensors. A person moving from location to location in the order of the numbers illustrated in Fig. 81 held both reflectors. When the person moved to one location, he stayed there for about six seconds, with the microwave reflector swinging forth and back to simulate a moving target. Data was collected in the on-bus computer. This is plotted in Fig. 82 thru Fig. 84.

Fig. 9 Object locations for system alignment

Fig. 10 PRADAR data for system alignment

Fig. 11 DRADAR data for system alignment

Fig. 12 LIDAR data for system alignment

The target parameters on the marked locations were extracted from the data and transformed to the bus coordinate system (FCS). Deviations are then calculated with the assumption that the marked locations are precise. The deviations are listed in Table 42 and plotted in Fig. 13. The average deviation of both distance and lateral position is less than 1m. This indicates the sensors are aligned well.

Fig. 13 Object locations reported by sensors

Table 11 Object locations reported by sensors

| Locat. # | LIDAR |  |  |  | Passenger Side Radar |  |  |  | Driver Side Radar |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Report |  | System (m) |  | Report |  | System (m) |  | Report |  | System (m) |  |  |
|  | R(H/L)1 | L2 | Ds5 | Ls5 | R3 |  | Ds5 | Ls5 | R3 |  | Ds5 |  | Ls5 |
| 1 | 8/00 | -9 | 10.32 | 0.23 | 340 | -19 | 10.43 | -0.66 | Missed |  |  |  |  |
| 2 | 16/29 | -9 | 20.85 | 0.40 | 690 | -11 | 21.10 | -0.59 | 680 | 30 | 20.76 |  | -0.26 |
| 3 | Missed |  |  |  | Missed |  |  |  | Missed |  |  |  |  |
| 4 | 16/30 | 23 | 20.86 | -2.80 | Missed |  |  |  | Missed |  |  |  |  |
| 5 | 24/00 | -8 | 30.80 | 0.46 | 910 | -5 | 27.81 | -0.77 | 930 | 18 | 28.4 |  | -0.04 |
| 6 | 23/33 | -58 | 29.85 | 5.46 | Missed |  |  |  | Missed |  |  |  |  |
| 7 | 24/29 | 42 | 31.09 | -4.54 | 860 | 45 | 26.17 | -3.41 | Missed |  |  |  |  |
| 8 | 31/88 | -7 | 40.56 | 0.53 | 1253 | -3 | 38.26 | -0.82 | 1290 | 12 | 39.38 |  | 0.04 |
| 9 | 30/90 | -57 | 39.38 | 5.53 | Missed |  |  |  | 1442 | -37 | 43.90 |  | 4.23 |
| 10 | 32/40 | 43 | 41.44 | -4.47 | 1375 | 41 | 41.84 | -4.48 | Missed |  |  |  |  |
| 11 | 39/30 | -7 | 50.22 | 0.7 | 1680 | -7 | 51.27 | -0.33 | 1680 | 22 | 51.23 |  | -1.27 |
| 12 | 38/80 | -57 | 49.44 | 5.7 | Missed |  |  |  | 1628 | -38 | 49.55 |  | 4.75 |
| 13 | 40/20 | 43 | 51.4 | -4.3 | 1752 | 27 | 53.39 | -3.53 | Missed |  |  |  |  |
| Deviation Range |  | - | -0.62 1.44 | 0.23 0.7 | - | - | -3.83 3.39 | -0.77 1.59 | - | - |  | -1.60 3.90 | -1.27 0.04 |
| Average Deviation |  | - | 0.52 | 0.49 | - | - | -0.09 | 0.05 | - | - |  | 0.54 | -0.43 |

- Range. ‘H/L’ are two bytes of Lidar report. The LSB of H-byte is 1.28m and that of L-byte is 0.01m.

- Lateral position.

- Radar range is an integer, multiples of 0.1ft.

- Azimuth angle. Radar azimuth angle is an integer, multiples of 0.002rad.

- Distance and lateral position in bus coordinate system.

### Host vehicle parameter

#### Offset

The following host vehicle (bus) parameters are biased in the collected data: steering angle, acceleration, brake pressure and wheel speed. The biased values were measured when the bus was stationary and are listed in the following Table.

Table 12 Host vehicle parameter biased values

| Parameter | Biased Value |
| --- | --- |
| Steering Angle | 7.36 |
| X Acceleration | 2.40 |
| Y Acceleration | 2.45 |
| Z Acceleration | 2.45 |
| Brake Pressure | 1.02 |
| Wheel Speed | 0.038574 |

Sensitivity

##### Steering angle sensor

The bus hand-wheel was turned counterclockwise as far as it would go, held for five seconds, then turned clockwise step by step. For each step the hand-wheel was rotated 120 degrees and held for five seconds (the last turn was less than 120 degrees). The steering angle sensor outputs when the hand-wheel was held are listed below:

8.715820 (anticlockwise end)

8.525391

8.374023

8.198242

8.012695

7.822266

7.631836

7.441406

7.255859

7.070312

6.894531

6.723633

6.562500

6.406250

6.254883

6.201172 (after the last turn)

Average sensitivity factor is 1.5 mV/degree. Average angle-to-voltage ratio is 687.6148 degrees/V.  The Steering ratio is 20.42:1 (for every degree of road wheel change requires 20.42 degrees of handwheel input). The wheel base is 279 inches.

##### Accelerometer

The accelerometer sensitivity is given by Summit Instruments as (unit: mV/g):

Table 13 Accelerometer sensitivity

|  | To X Acceleration | To Y Acceleration |
| --- | --- | --- |
| X Sensitivity | 1302.31 | 8.76 |
| Y Sensitivity | -13.07 | 1299.00 |
| Z Sensitivity | -3.25 | 2.97 |

The accelerometer was not calibrated on the bus.

##### Brake pressure

The brake pressure transducer output is proportional to the pressure. The sensitivity factor is 50mV/psi. The pressure range is 0-100psi. The corresponding output range is 1-6V.

Data storage

This project has generated large amounts of data.  Currently there are over 200 gigabytes of video and sensor data.  The data was initially stored on one computer with three large hard drives but soon the data storage reached its maximum capacity. A new storage solution was therefore developed. This new storage method is built using a RAID. It currently has 800 gigabytes of storage capacity which will allow this project to collect full data from three buses for one year. The RAID can be expanded to 1.5 terabytes if necessary.  Later in the project we expect to perform more selective data collection which will reduce the rate of data collection.  This will be done by only collecting data when the warning algorithm has given an alert. The new storage system is connected through the internet to allow users obtain data online. The RAID-based storage system has proven to be the most convenient and economical solution to our data storage needs.

References

- Denso Lidar documents, Denso Inc.

- Lenz, R.K.; Tsai, R.Y. Techniques for calibration of the scale factor and image center for high accuracy 3-D machine vision metrology, Pattern Analysis and Machine Intelligence, IEEE Transactions on , Volume: 10 Issue: 5 , Sept. 1988, Page(s): 713 -720.

- Bacakoglu, H.; Kamel, M.S. A three-step camera calibration method. IEEE Transactions on Instrumentation and Measurement, vol.46, (no.5), IEEE, Oct. 1997. p.1165-72.

- Weng, J.; Cohen, P.; Herniou, M. Camera calibration with distortion models and accuracy evaluation. IEEE Transactions on Pattern Analysis and Machine Intelligence, vol.14, (no.10), Oct. 1992. p.965-80.

- Wu, L.D., Computer Vision, Fudan University, China, Dec. 1993.

| Appendix VI: | Suggestions on Other Display Modalities |
| --- | --- |

Table 14 Audible warnings feedback from operators and trainers

| Requirement from Operators/Trainers | Requirement |
| --- | --- |
| T | Suggested an earbud since it would provide a private audio warning, but recognized that operators would probably not be too keen on wearing them. |
| T | Sound was perceived to be a more effective option than vision as a knowledgeable operator would not have to look at anything to detect the warning. |
| T | Operators are trained to look down when an audible alarm trips as they often signal a mechanical problem. Instrument panel lights are used to identify the problem. |
| D | Ambient sound levels within a bus can vary due to passenger load. |
| D | Operators find some existing warnings very annoying, especially when repeated false alarms occur. One example frequently cited was the rear door buzzer. |
| D | Speakers placed behind the operator's head (e.g., one on each side for directional information) were uniformly rejected. This location was perceived to be too startling. Music was identified as the only acceptable audio source from this location |
| D | One operator proposed a dash-mounted display similar to an on-board radar screen for airplanes. |
| D | Head-Up Displays (HUDs) were voluntarily suggested. Operators were aware that the cost would likely be too great. For a detailed discussion of additional problems with HUDs in transit buses see [Error: Reference source not found]. |
| D | Dash lights already flicker often during normal driving (e.g., when braking, retarder activation, etc.). |
| D | Operators with night runs indicated that there is already too much illumination in the cab and were not enthusiastic about additional light sources. The ability to dim the illumination level or even shut off visual warnings was requested. Redundant warnings in other modalities (e.g., audible) were recommended for night driving. |
| DT | Experienced operators often downplayed the value of dash-mounted visual warnings as they rarely look at their instrument cluster. |

| D | Audible warnings that change pitch as the danger increases were suggested. |
| --- | --- |
| D | Chimes or other subtle, pleasant sounds were suggested as ways to provide alerts (prior to full-blown warnings). |
| D | Ramping up the volume or pitch was suggested as a way to not startle the operator. |
| D | Concern over having passengers mimic the sound of any warning |

Table 15 Tactile warnings feedback from operators and trainers

| Requirement from Operators/Trainers | Requirement |
| --- | --- |
| DT | Seat vibrations were roundly described as not worthwhile. One operator commented, "After 8 hours I don't have any idea what's going on down there." |

Table 16 Vehicle control feedback from operators and trainers

| Requirement from Operators/Trainers | Requirement |
| --- | --- |
| D | Operators voluntarily suggested longitudinal control actions by the bus. The existing interlock was subsequently mentioned as a potential method. The interlock activates whenever the rear door is opened by applying brake pressure to the two rear wheels. This prevents the bus from rolling away while passengers are loading. In the past, the interlock would be triggered if the rear door was opened while the bus was in motion. Maintenance has prevented the rear door from opening while in motion due to concerns over brake wear. Experienced operators described the braking action as being smooth enough to prevent falls, yet fast enough to bring the bus to rest. |

## Audible component design

One of the more interesting operator suggestions was to use a non-traditional auditory sound for the alert level (in this case a chime or a clock tick). This level would be the point at which the system indicates that there might be a target that could lead to a critical threat. The use of a chime or some other semi-pleasant notification earcon for this threshold is important, as this event will be somewhat frequent. A comparable warning would be the soft thunderclap or "clink" sound used by supermarkets to warn patrons in the produce section of an upcoming water spray. The sound is unique enough for patrons and employees to detect, yet is not obtrusive enough to annoy those present.

For cases of true critical events (i.e., chances are high that a crash will occur) a more salient and obtrusive earcon was recommended. Furthermore, the warning should not be binary in nature. The volume should ramp up as the threat level increases. In reality, the inclusion of a volume knob will allow operators to have either earlier or later perception, as the early ramping phase will be highly affected by the knob setting.

Speaker positioning will be important, as it will be necessary to provide a clear sound to the operator without being readily detectable to the passengers. Obviously the trainer suggestion of ear buds would be the easiest solution but the driving population would not accept these. Speakers behind the operator's head are another logical solution, but again, this was not popular with the operators. The remaining options are in front of the operator as side mounted locations will be directed towards passengers in the front seats and the sounds will not have good spatial mapping to the forward threat [1]. Final speaker placement will likely be done during the DVI installation process as the geometry of the cab is complex.

## References

- Tan, A. K., & Lerner, N. (1996). Acoustic Localization of In-Vehicle Crash Avoidance Warnings as a Cue to Hazard Direction (DOT HS 808 534). Washington, DC: National Highway Traffic Safety Administration.

Appendix VII: FCWS Survey Questions

Forward Collision Warning System (FCWS) Evaluation Survey Questions

SamTrans February 2002

We would like to ask you some questions regarding your opinion of the FCWS. We will not be recording your identity and this information will not be associated with you or be used as a means of evaluating your performance. We are only interested in evaluating the system.

Your participation is voluntary. You are free to refuse to take part. You may refuse to answer any question and may stop taking part in the study at any time. Whether or not you participate in this research will have no bearing on you standing in your job.

Background information:

How long have you been driving buses?

Probing Questions – used as required:

Did the system function the way that you thought it would?

Please describe any instances where you felt that you should have received a warning but didn’t?

Please describe any instances where you received a warning and felt that you shouldn’t have?

Do you like the system?

Did you find the system easy to use?

How long do you think you would need to become comfortable with the system?

When did the system provide you with the most assistance?

Would you like to drive with the system the way it currently is?

If you could change/add one feature what would it be?

Questions/comments

Appendix VIII: FCWS Evaluation Questionnaire

Forward Collision Warning System (FCWS) Evaluation Questionnaire

SamTrans April 2002

We would like to ask you some questions regarding your opinion of the FCWS. We will not be recording your identity and this information will not be associated with you or be used as a means of evaluating your performance. We are only interested in evaluating the system.

Your participation is voluntary. You are free to refuse to take part. You may refuse to answer any question and may stop taking part in the study at any time. Whether or not you participate in this research will have no bearing on you standing in your job.

Background information:

How long have you been driving buses?   ___________

Approximately how many hours have you driven the bus with the FCWS on?_________

Did you receive any training prior to using the FCWS?___________

General Assessment:

- Please describe the system and how it works the way that you would to another operator that has not yet seen or used the system.

______________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

For the following questions, please rate how well the system performs:

| How easy is the system to use overall? | (not easy) 1 2 3 4 5 (very easy) |
| --- | --- |
| How much do you like the system overall? | (not at all) 1 2 3 4 5 (a lot) |
| How well do you think the warnings conveyed a sense of urgency? | (not at all) 1 2 3 4 5 (a lot) |
| If you had more time with the system, would you like it more? | (no) 1 2 3 4 5 (yes) |
| Do you think that they system is beneficial in terms of increasing your safety? | (not at all) 1 2 3 4 5 (extremely) |
| How annoying was the system? | (not at all) 1 2 3 4 5 (extremely) |
| How distracting was the system? | (not at all) 1 2 3 4 5 (extremely) |